Assembly instructions FIPT cell

# Revision table

| Date | Person | Rev# | Comment |
| --- | --- | --- | --- |
| 191205 | DB | 001 | First version |

Note: create new file for each revision. Increment revision number in file name

Note: save a new file/revision before starting to change stuff

# Summary

Necessary items:

- 3x M5x40mm hex socket bolts

- 3d printed parts: (Typically: PLA, 250um layer height, 500um nozzle diameter)

  - 071_0100_Base.stl

  - 071_0101_Top.stl

  - 071_0102_Clamp.stl

  - 071_0105_ScrewCap.stl

- 20mm wide Kapton tape

- >= 20mm wide double sided tape

- 2-3mm thick EVA foam rubber sheet (from arts and crafts store)

Assembly steps:

- Print all items

- Add foam to Top part and Base part as in pictures below.

These foam parts consists of 3 layers:

  - Double sided tape

  - Foam sheet

  - Kapton tape

Make sure they are 20mm wide and that Kapton covers all foam (such that no electrolyte is absorbed).

Note: The foam parts are necessary to avoid touching of the edges of the crossing electrodes. Please follow the order/orientation of the SOP exactly. Otherwise, shorting and other problems can regularly occur!

# Assembly pictures